Culvert and Structure Rating Codes

| Code | Culverts-any CMP or CMAP of any size or other material structure under 48” | Structures – Anything 48”or greater and any material. |
| --- | --- | --- |
| 5 | New or like new condition, structurally sound and functionally adequate | New or like new condition, structurally sound and functionally adequate |
| 4 | Some deterioration i.e., minor rust, minor damage but structurally sound and functionally adequate. | Some deterioration i.e., minor rust, minor damage, weathered planks or piling, minor cracking in concrete but structurally sound and functionally adequate. |
| 3 | Advanced deterioration i.e., rust with minor pinholes, major damage to ends and waterway issues starting to develop. Too short or low. | Advanced deterioration i.e., rust with minor pinholes, major damage to ends, deck plank or piling issues, minor cracking with spalling and waterway issues starting to develop. |
| 2 | Significant deterioration i.e., major rust with large holes, smashed ends that affect performance and significant waterway issues. | Significant deterioration i.e., major rust with large holes, Deformation in pipe shape due to rust. Major plank or pile damage. Severe cracking and spalling that may affect structural performance. Significant waterway issues. |
| 1 | Critical. Very poor conditions indicate possible failure. Needs to be assessed for closing and possible immediate action. | Critical. Very poor conditions indicate possible failure. Needs to be assessed for closing and possible immediate action. |
| 0 | All or part of culvert is inaccessible for assessment or a rating cannot be assigned. | Structure is closed. |